Table of Contents

# Technical Analysis of the UR3 ROS2 Pick-and-Place System

## A Dissertation-Quality Case Study in ROS 2 Robotic Manipulation

Repository: darshmenon/UR3_ROS2_PICK_AND_PLACE
Platform: ROS 2 Jazzy · Gazebo Harmonic · Ubuntu 24.04
Primary Stack: C++ (79.3%) · Python (16.0%) · CMake (4.3%)
Analysis Date: May 2026

## Table of Contents

- Executive Summary

- System Architecture Overview

- Hardware Model: UR3 Robot and Robotiq 2F-85 Gripper

- Package Structure and Dependency Graph

- Package Deep Dives

  - ur_description

  - ur_gazebo

  - moveit_config

  - robotiq_2f_85_gripper_visualization

  - ur_moveit_demos

  - ur_mtc_demos and ur_mtc_pick_place_demo

  - ur_interfaces

  - ur_system_tests

- ROS 2 Communication Architecture

  - Topics

  - Services

  - Actions

- ros2_control Integration

- MoveIt 2 Integration

- MoveIt Task Constructor (MTC) Pipeline

- Gazebo Harmonic Integration

- Kinematic Chain and Motion Planning

- Pick-and-Place Workflow

- Gripper Control Architecture

- Launch Architecture

- Controller Configuration

- Data Flow and Runtime Behavior

- Software Engineering Analysis

- Comparison to Industrial Systems (UR5e)

- Alternative Implementation Approaches

- Emerging Capabilities and Future Directions

- Conclusions

## 1. Executive Summary

This repository implements a complete pick-and-place robotic manipulation system using a Universal Robots UR3 arm paired with a Robotiq 2F-85 two-finger parallel gripper, fully simulated in Gazebo Harmonic and planned through MoveIt 2 running under ROS 2 Jazzy. The system demonstrates two planning paradigms: direct MoveGroup API calls for simple trajectory planning, and the MoveIt Task Constructor (MTC) framework for compositional multi-stage task planning.

The codebase is architecturally layered: a URDF/xacro robot description layer, a physics simulation layer (Gazebo Harmonic), a real-time control layer (ros2_control with hardware interfaces), a motion planning layer (MoveIt 2 with OMPL and PILZ planners), a task planning layer (MTC), and an application layer (Python scripting and C++ demo nodes). This separation of concerns mirrors best practices in industrial robotics middleware and positions the system as a solid foundation for research into perception-driven, autonomous manipulation.

Key design decisions analyzed in this report include:

- The fixed mimic-joint approach for the Robotiq gripper, which sidesteps a Gazebo Harmonic limitation regarding dynamic mimic joint simulation

- The choice of position_controllers/GripperActionController for gripper control, decoupled from the arm’s JointTrajectoryController

- A bootstrap delay pattern (~40s) to account for Gazebo physics initialization before controller activation

- Dual planning pipeline support (OMPL for free-space planning, PILZ for Cartesian LIN/CIRC motions)

- The progression from scripted Python pick-and-place to MTC-based compositional task specification

The repository also contains early-stage but conceptually important extensions: a SAC-based reinforcement learning training pipeline in MuJoCo with Gazebo transfer, depth-camera-based point cloud grasp estimation, YOLO-based object detection, and an Ollama LLM-backed natural language task planner. These extensions are not yet production-integrated but represent the system’s intended long-term trajectory.

## 2. System Architecture Overview

The system can be understood as five horizontal layers communicating through ROS 2’s DDS (Data Distribution Service) middleware.

┌─────────────────────────────────────────────────────────────┐
│  APPLICATION LAYER                                          │
│  arm_gripper_loop_controller.py · custom_zigzag_motion      │
│  ur_mtc_pick_place_demo node                                │
├─────────────────────────────────────────────────────────────┤
│  TASK PLANNING LAYER                                        │
│  MoveIt Task Constructor (MTC)                              │
│  Stages: CurrentState → OpenGripper → Approach → Pick →     │
│          Lift → Move → Place → Retreat → CloseGripper       │
├─────────────────────────────────────────────────────────────┤
│  MOTION PLANNING LAYER                                      │
│  MoveIt 2 (move_group node)                                 │
│  Planners: OMPL (RRTConnect) · PILZ (LIN, CIRC, PTP)       │
│  Planning Scene · Kinematics (KDL / TracIK)                 │
├─────────────────────────────────────────────────────────────┤
│  CONTROL LAYER (ros2_control)                               │
│  JointTrajectoryController (arm_controller)                 │
│  GripperActionController (gripper_controller)               │
│  JointStateBroadcaster                                      │
├─────────────────────────────────────────────────────────────┤
│  PHYSICS / HARDWARE LAYER                                   │
│  Gazebo Harmonic (gz-ros2-control hardware interface)       │
│  URDF / SRDF model of UR3 + Robotiq 2F-85                   │
└─────────────────────────────────────────────────────────────┘

Horizontal data flow (simplified):

Gazebo Sim ──/joint_states──► robot_state_publisher ──/tf──► MoveIt 2
                                                               │
move_group ◄──Action Goals──── MTC / Application Layer        │
     │                                                         │
     └──/arm_controller/follow_joint_trajectory──► ros2_control
                                                       │
                                                   Gazebo Sim

At runtime, Gazebo Harmonic runs the physics engine and exposes the hardware interface through the gz_ros2_control plugin. The controller_manager spawns and manages the three controllers. The robot_state_publisher converts /joint_states into /tf transforms. MoveIt 2’s move_group node uses the TF tree and the URDF/SRDF to answer motion planning requests arriving from application code, either directly through the MoveGroup C++ API or indirectly through MTC’s stage-based planner.

## 3. Hardware Model: UR3 Robot and Robotiq 2F-85 Gripper

### 3.1 UR3 Specifications and Kinematic Structure

The Universal Robots UR3 is a 6-DOF collaborative serial manipulator with a payload rating of 3 kg, a reach of 500 mm, and a mass of approximately 11.2 kg. It is designed for light-duty assembly, dispensing, and pick-and-place tasks in human-collaborative environments. Its kinematic configuration is a non-spherical wrist design, which complicates analytical inverse kinematics but remains within the class of problems that iterative solvers (KDL, TracIK) and numerical sampling-based planners (OMPL) handle robustly.

Denavit-Hartenberg Parameters (UR3, modified DH convention):

| Joint | a (m) | d (m) | α (rad) | θ offset (rad) |
| --- | --- | --- | --- | --- |
| 1 (shoulder_pan) | 0.0 | 0.1519 | π/2 | 0 |
| 2 (shoulder_lift) | −0.24365 | 0.0 | 0 | 0 |
| 3 (elbow) | −0.21325 | 0.0 | 0 | 0 |
| 4 (wrist_1) | 0.0 | 0.11235 | π/2 | 0 |
| 5 (wrist_2) | 0.0 | 0.08535 | −π/2 | 0 |
| 6 (wrist_3) | 0.0 | 0.0819 | 0 | 0 |

The forward kinematics chain is computed as T_base_6 = T01 · T12 · T23 · T34 · T45 · T56, where each transformation matrix follows the standard DH product: T_n−1,n = Rot_z(θ_n) · Trans_z(d_n) · Trans_x(a_n) · Rot_x(α_n).

Joint name mapping in ROS 2:

| ROS 2 Joint Name | Physical Joint | Range |
| --- | --- | --- |
| shoulder_pan_joint | J1 | ±360° |
| shoulder_lift_joint | J2 | ±360° |
| elbow_joint | J3 | ±360° |
| wrist_1_joint | J4 | ±360° |
| wrist_2_joint | J5 | ±360° |
| wrist_3_joint | J6 | ±360° |

In the URDF, these joints are declared as continuous revolute joints with effort and velocity limits. The joint limits in the SRDF further constrain them during MoveIt planning to prevent self-collision configurations.

### 3.2 Robotiq 2F-85 Gripper

The Robotiq 2F-85 is an adaptive, parallel two-finger electric gripper with an 85 mm stroke, capable of both parallel and encompassing grasps. Its mechanism includes six joints per finger arranged as a kinematic chain that enforces a coupling constraint: the inner knuckle and outer knuckle joints follow the primary finger joint through a 4-bar linkage mechanism.

In a real Robotiq driver, this coupling is enforced by the physical hardware. In simulation, it must be modeled via URDF mimic joints. This is the central challenge in this repository.

Gripper joint hierarchy:

finger_joint (primary, commanded)
├── left_inner_knuckle_joint   (mimic, multiplier=1.0)
├── left_inner_finger_pad_joint (mimic, multiplier=−1.0)
└── right_outer_knuckle_joint  (mimic, multiplier=1.0)
    ├── right_inner_knuckle_joint (mimic, multiplier=1.0)
    └── right_inner_finger_pad_joint (mimic, multiplier=−1.0)

Only finger_joint receives commanded positions. Mimic joints follow via the <mimic> URDF tag. In Gazebo Harmonic, physics-level enforcement of mimic constraints (rather than controller-level) makes the simulation both more stable and more faithful to the hardware mechanism.

## 4. Package Structure and Dependency Graph

The workspace is organized into ten packages at the top level:

UR3_ROS2_PICK_AND_PLACE/
├── ur_description/                 # URDF/xacro robot model
├── robotiq_2f_85_gripper_visualization/  # Gripper URDF + visualization
├── ur_gazebo/                      # Gazebo simulation launch + world files
├── moveit_config/                  # MoveIt 2 SRDF, kinematics, OMPL config
├── ur_moveit_demos/                # C++ MoveGroup API demonstrations
├── ur_mtc_demos/                   # Basic MTC stage demonstrations
├── ur_mtc_pick_place_demo/         # Full MTC pick-and-place pipeline
├── ur_interfaces/                  # Custom ROS 2 message/service/action definitions
├── ur_system_tests/                # Integration test scripts (Python)
├── src/                            # Additional source (grasp node, RL training)
├── testing/                        # Experimental / in-progress features
├── images/                         # Documentation images and GIFs
├── CONCEPTS.md                     # Architecture concepts documentation
├── INSTALL.md                      # Installation guide
└── README.md                       # Project entry point

Package dependency graph:

ur_description ◄──────────────────────── robotiq_2f_85_gripper_visualization
      │                                                  │
      ▼                                                  ▼
  ur_gazebo ◄─────────────────────── moveit_config ◄─── ur_moveit_demos
      │                                    │
      │                                    ▼
      └──────────────────────────► ur_mtc_demos
                                         │
                                         ▼
                                 ur_mtc_pick_place_demo ◄── ur_interfaces
                                         │
                                         ▼
                                   ur_system_tests

Build dependencies for the full system:

| Dependency | Purpose |
| --- | --- |
| ros2_control | Control framework core |
| ros2_controllers | JointTrajectoryController, GripperActionController |
| controller_manager | Dynamic controller lifecycle management |
| gz_ros2_control | Gazebo Harmonic ↔ ros2_control bridge |
| moveit_ros_planning_interface | C++ MoveGroup API |
| moveit_task_constructor_core | MTC framework |
| robot_state_publisher | URDF → /tf |
| joint_state_publisher_gui | Manual joint control in RViz |
| gz-sim | Gazebo Harmonic physics engine |
| rviz2 | 3D visualization |

## 5. Package Deep Dives

### 5.1 ur_description

What it does:
This package contains the complete robot description in URDF/xacro format. It models the UR3 arm’s geometry, inertial properties, collision meshes, and visual meshes, and integrates the Robotiq gripper at the tool flange.

Key files:

| File | Role |
| --- | --- |
| urdf/ur3.urdf.xacro | Top-level robot macro, assembles arm + gripper |
| urdf/ur.urdf.xacro | Parametric UR arm definition (supports ur3/ur5/ur10) |
| urdf/ur.ros2_control.xacro | Declares ros2_control hardware interfaces |
| meshes/ur3/ | STL collision and DAE visual meshes |
| config/ur3_initial_positions.yaml | Default joint home positions |
| launch/view_ur.launch.py | Launches RViz for model visualization |

Why it exists:
URDF is the universal robot description language in ROS. It is consumed by robot_state_publisher (to compute transforms), MoveIt (to build collision geometry), Gazebo (to spawn the physics model), and ros2_control (to enumerate hardware interfaces).

The ros2_control xacro:
The file ur.ros2_control.xacro is architecturally important. It declares the hardware plugin — for simulation this is gz_ros2_control/GazeboSimSystem. It also declares state_interface (position, velocity) and command_interface (position) for each joint. In real-hardware deployment, swapping the hardware plugin to ur_robot_driver/URPositionHardwareInterface would be the only file change needed to transition from simulation to the physical robot.

Alternative approaches:
A common alternative is to use the ur_robot_description package from the official Universal Robots ROS 2 driver repository, which provides a more configurable xacro pipeline with support for tool calibration, flange offsets, and safety limits. This project instead uses a standalone, simplified description optimized for simulation clarity and rapid prototyping.

Strengths: Self-contained, easy to modify, clear separation between visual and collision geometry.
Weaknesses: Not synchronized with Universal Robots’ official URDF updates; may require manual porting for new UR firmware versions.

### 5.2 ur_gazebo

What it does:
This is the launch and world-file package that orchestrates the entire simulated runtime environment. It spawns the robot model in Gazebo, loads the ros2_control controller manager, activates controllers, and optionally launches MoveIt 2.

Key files:

| File | Role |
| --- | --- |
| launch/ur.gazebo.launch.py | Master launch file (Gazebo + controllers + MoveIt) |
| worlds/ur_pick_place.world | SDF world: table, object, lighting |
| config/ur_controllers.yaml | Controller parameter definitions |
| config/ros2_control.yaml | Hardware interface plugin mapping |

Launch sequence within ur.gazebo.launch.py:

- Parse launch arguments (ur_type, gripper, world_file, etc.)

- Load robot URDF via xacro into the /robot_description parameter

- Start robot_state_publisher node

- Start Gazebo Harmonic (gz_sim) with the specified world file

- Spawn the robot entity into Gazebo via /spawn_entity service

- Start controller_manager with the controller config YAML

- Use spawner nodes to activate: joint_state_broadcaster, arm_controller, gripper_controller

- (Optional) Launch MoveIt 2’s move_group node

The 40-second bootstrap delay:
Controllers are spawned with a delay argument (approximately 40 seconds) to allow Gazebo’s physics engine and the gz-ros2-control plugin to fully initialize before the controller manager attempts to connect. This is a pragmatic solution to a race condition inherent in Gazebo Harmonic’s asynchronous initialization. In production systems, a more robust approach would use lifecycle nodes and a controller_manager watchdog service.

Gripper parametrization:
The launch file accepts a gripper argument with valid values of robotiq_2f_85, robotiq_2f_140, onrobot_rg2, and onrobot_rg6. All four grippers use position_controllers/GripperActionController for the single commanded joint, because regardless of gripper model, the mimic-joint architecture reduces the control problem to commanding a single finger_joint.

World file (ur_pick_place.world):
The SDF world file defines a flat tabletop surface at a fixed height (approximately z = 0.77 m), a pick object (typically a small box with visual and collision geometry), ambient lighting, and a ground plane. The object’s pose defines the ground truth pick target used by the MTC planner. Real systems would derive this from a perception pipeline; in this simulation it is hardcoded.

### 5.3 moveit_config

What it does:
This package contains the complete MoveIt 2 configuration for the UR3 + Robotiq 2F-85 system, typically generated by the MoveIt Setup Assistant and then manually tuned.

Key files:

| File | Role |
| --- | --- |
| config/ur3_robotiq.srdf | Semantic Robot Description Format: planning groups, named states, disabled collisions |
| config/kinematics.yaml | IK solver selection and parameters |
| config/ompl_planning.yaml | OMPL planner library configuration |
| config/pilz_industrial_motion_planner_planning.yaml | PILZ planner parameters |
| config/moveit_controllers.yaml | Controller-to-planning-group mapping |
| config/joint_limits.yaml | Per-joint velocity/acceleration limits for trajectory generation |
| launch/move_group.launch.py | Launches the move_group node with all parameters |
| launch/moveit_rviz.launch.py | Launches RViz with MoveIt 2 plugin |

SRDF planning groups:
The SRDF defines two planning groups:

- arm — Contains all six UR3 joints (shoulder_pan_joint through wrist_3_joint). This group is used for free-space motion planning.

- gripper — Contains finger_joint only. This group is used for open/close gripper commands.

Named states defined in the SRDF include home (arm fully retracted upright), ready (arm at a nominal approach height), and open/closed for the gripper. These named states are used extensively by MTC stages to define approach and retreat configurations.

Disabled collision pairs:
The SRDF disables self-collision checking for pairs of links that are geometrically adjacent (always in contact or very close). This is critical for planning performance; without these disabled pairs, OMPL’s collision checker would spend significant time evaluating contacts that can never be collisions. Typical disabled pairs include base-link to shoulder links, and all combinations of adjacent gripper links.

Kinematics configuration:
kinematics.yaml selects the IK solver per planning group: - For the arm group: KDL (Kinematics and Dynamics Library) by default, optionally TracIK for faster convergence on the UR3’s non-spherical wrist. - Timeout: 0.05 seconds; attempts: 3. These are conservative values suitable for real-time replanning.

KDL implements a numerical Jacobian-pseudoinverse iterative solver. TracIK uses a sequential quadratic programming (SQP) approach that handles joint limits better. For the UR3 specifically, the non-spherical wrist (no coincident wrist joint origins) means there is no closed-form analytical solution under the standard Pieper condition — iterative solvers are necessary.

OMPL configuration:
The OMPL planner configuration defines a library of sampling-based planners:

| Planner | Full Name | Characteristic Use |
| --- | --- | --- |
| RRTConnect | Rapidly-exploring Random Trees (bidirectional) | Default; fast for most configurations |
| RRT | Unidirectional RRT | Simple environments |
| RRTstar | Asymptotically optimal RRT | When path quality matters |
| TRRT | Transition-based RRT | Narrow passages |
| LBTRRT | Lower Bound Tree RRT | Planning cost awareness |
| EST | Expansive Space Trees | Dense environments |
| SBL | Single-query Bidirectional Lazy | Fast approximate planning |
| KPIECE1 | Kinodynamic Planning by Interior-Exterior Cell Exploration | High-DOF spaces |
| BKPIECE1 | Bidirectional KPIECE | General purpose |

The default planner for the arm group is RRTConnect, which is the practical gold standard for 6-DOF manipulators due to its bidirectional tree growth and ability to rapidly find feasible paths in joint space.

PILZ configuration:
The PILZ Industrial Motion Planner is an alternative planner plugin in MoveIt 2 that provides deterministic, geometry-based trajectory generation:

- PTP (Point-to-Point): Trapezoidal velocity profiles in joint space; all joints reach target simultaneously.

- LIN (Linear): Straight-line Cartesian end-effector motion; used for approach/retreat phases in pick-and-place.

- CIRC (Circular): Arc motion through an auxiliary waypoint; used for circular assembly paths.

The Zig-Zag demo (custom_zigzag_motion) in ur_moveit_demos uses PILZ’s LIN planner to generate a sequence of linear Cartesian segments, demonstrating industrial-style teach-and-repeat motion.

### 5.4 robotiq_2f_85_gripper_visualization

What it does:
Provides the URDF/xacro model of the Robotiq 2F-85 gripper as a standalone subpackage, along with a visualization launch file.

Key files:

| File | Role |
| --- | --- |
| urdf/robotiq_2f_85.urdf.xacro | Gripper geometry, inertia, mimic joint definitions |
| launch/test_2f_85_model.launch.py | Standalone RViz visualization of the gripper |

Mimic joint implementation:
The critical design decision here is the <mimic> tag approach for Gazebo Harmonic compatibility. In earlier Gazebo versions (Classic / Fortress), mimic joints required a dedicated libgazebo_ros_mimic_joints_plugin to enforce constraints at the physics level, or were handled by the ros2_control mimic joint interface. In Gazebo Harmonic, physics-level enforcement of <mimic> constraints is built in when the SDF model is properly structured.

The xacro file translates URDF <mimic> tags into the appropriate SDF format during the xacro-to-URDF-to-SDF pipeline. The key parameter is that only finger_joint is declared with a ros2_control command_interface; all other joints have only state_interfaces. The controller therefore drives only the primary joint, and the physics engine enforces coupling.

Alternative approaches:
An alternative (used in some earlier ROS repositories) is to use a GripperJointInterface that explicitly commands all mimic joints simultaneously to maintain synchrony. This is more computationally expensive and introduces potential for numerical drift between joints. The mimic enforcement approach is superior in Gazebo Harmonic.

### 5.5 ur_moveit_demos

What it does:
Contains C++ nodes that demonstrate direct use of the MoveIt 2 MoveGroupInterface API for arm and gripper control.

Key executables:

| Executable | Description |
| --- | --- |
| custom_zigzag_motion | Cartesian zig-zag via PILZ LIN planner |
| move_to_pose | Move arm to target Cartesian pose |
| joint_goal | Move arm to target joint configuration |

custom_zigzag_motion — Architectural Analysis:

This node illustrates the most common pattern for MoveIt 2 programmatic control:

// 1. Initialize MoveGroupInterface for the "arm" planning group
auto move_group = std::make_shared<moveit::planning_interface::MoveGroupInterface>(
    node, "arm");

// 2. Set planner ID to PILZ LIN for Cartesian segments
move_group->setPlannerId("LIN");

// 3. Set velocity and acceleration scaling
move_group->setMaxVelocityScalingFactor(0.3);
move_group->setMaxAccelerationScalingFactor(0.3);

// 4. Define waypoints in Cartesian space
std::vector<geometry_msgs::msg::Pose> waypoints;
// ... populate waypoints for zig-zag pattern ...

// 5. Compute Cartesian path
moveit_msgs::msg::RobotTrajectory trajectory;
double fraction = move_group->computeCartesianPath(
    waypoints, 0.01, 0.0, trajectory);

// 6. Execute if path fraction is sufficient
if (fraction > 0.95) {
    move_group->execute(trajectory);
}

The computeCartesianPath call invokes the IK solver iteratively at small step intervals (0.01 m) along the desired Cartesian path, building a joint-space trajectory that tracks the Cartesian waypoints. The fraction return value indicates what percentage of the path was successfully planned; values below 0.95 indicate kinematic singularities or joint limits along the path.

Why C++ rather than Python:
The C++ MoveGroupInterface is substantially lower latency than the Python bindings (which use a pybind11 wrapper) because it avoids Python GIL overhead and message serialization costs. For high-frequency replanning scenarios or time-critical applications, C++ is the appropriate choice. The demos use C++ to model this industrial best practice.

### 5.6 ur_mtc_demos and ur_mtc_pick_place_demo

What it does:
These two packages implement pick-and-place planning using the MoveIt Task Constructor (MTC) framework. ur_mtc_demos contains basic stage demonstrations; ur_mtc_pick_place_demo contains the complete, executable pick-and-place pipeline.

MoveIt Task Constructor conceptual model:
MTC decomposes a complex manipulation task into a graph of planning stages. Each stage has a type that determines its planning semantics:

| Stage Type | Direction | Example Use |
| --- | --- | --- |
| Generator | None (creates states) | CurrentState, GeneratePose, GenerateGraspPose |
| Propagator (forward) | Current → Next | MoveTo, MoveRelative |
| Propagator (backward) | Next → Current | Connect planners |
| Connector | Connects from and to | Bidirectional bridges |
| Container (serial) | Ordered sequence | Overall task |
| Container (parallel) | Multiple alternatives | Fallbacks, Alternatives |

The power of MTC lies in its ability to propagate planning constraints backwards. For example, the grasp pose must be consistent with both the pre-grasp approach (planning forward) and the post-grasp lift (planning backward). MTC’s stage graph resolves these coupled constraints through a cost-minimizing solution tree.

Full pick-and-place stage pipeline:

Task (Serial Container)
│
├── Stage 1: CurrentState [Generator]
│   └── Captures current joint positions and planning scene
│
├── Stage 2: OpenGripper [MoveTo, forward]
│   └── Plans gripper to "open" named state
│
├── Stage 3: MoveToPreGrasp [Connect]
│   └── Plans arm from current to pre-grasp approach pose
│
├── Stage 4: GenerateGraspPose [Generator]
│   └── Samples candidate grasp poses around object
│
├── Stage 5: Approach [MoveRelative, forward]
│   └── Linear approach toward object (PILZ LIN, −z direction)
│
├── Stage 6: CloseGripper [MoveTo, forward]
│   └── Plans gripper to "closed" named state; updates planning scene
│
├── Stage 7: AttachObject [ModifyPlanningScene]
│   └── Attaches object collision box to gripper frame
│
├── Stage 8: Lift [MoveRelative, forward]
│   └── Linear lift above table (+z direction)
│
├── Stage 9: MoveToPlace [Connect]
│   └── Plans arm from lift pose to place target
│
├── Stage 10: PlaceDown [MoveRelative, forward]
│   └── Linear descent to place surface
│
├── Stage 11: OpenGripper [MoveTo, forward]
│   └── Plans gripper to "open" state; releases object
│
├── Stage 12: DetachObject [ModifyPlanningScene]
│   └── Removes object from gripper frame, adds to world
│
└── Stage 13: Retreat [MoveRelative, forward]
    └── Linear retreat from place pose

Planning scene management:
A critical aspect of MTC is the planning scene server, which maintains a representation of the world state (collision objects, attached objects, robot state). Before running the MTC demo, a planning scene server node must add the table and pick object as collision boxes. The ur_mtc_pick_place_demo includes a planning_scene_server.cpp node that publishes these collision objects on startup.

The robot.sh orchestration script:
The bash script ur_mtc_pick_place_demo/scripts/robot.sh coordinates the multi-terminal launch sequence: 1. Launch Gazebo simulation (ur.gazebo.launch.py) 2. Wait for controllers to be active (polls ros2 control list_controllers) 3. Launch MoveIt 2 (move_group.launch.py) 4. Adjust Gazebo camera position via service call 5. Launch planning scene server 6. Launch the MTC pick-and-place demo node

This sequential orchestration ensures each component is ready before the next depends on it. A more robust alternative would use ROS 2 lifecycle nodes and managed launch with readiness conditions, but the shell script approach is pragmatic and transparent.

### 5.7 ur_interfaces

What it does:
Defines custom ROS 2 message, service, and action types specific to this system.

Typical contents for a system of this type:

| Interface | Type | Purpose |
| --- | --- | --- |
| PickAndPlaceGoal.action | Action | Specifies pick pose, place pose, and execution flags |
| GraspTarget.srv | Service | Request a grasp pose for a named object |
| ManipulationStatus.msg | Message | Execution status and error codes |

Having a dedicated interfaces package is a best practice in ROS 2: it ensures that message definitions are independent of their implementation, enabling multiple packages to depend on the same interface types without circular dependencies.

### 5.8 ur_system_tests

What it does:
Contains integration-level Python test scripts and validation utilities.

Key script — arm_gripper_loop_controller.py:
This is the primary Python orchestration script for executing the pick-and-place workflow. It uses the ROS 2 Python client library (rclpy) with action clients to command both the arm and gripper controllers directly, bypassing MoveIt 2.

Architectural pattern:

import rclpy
from rclpy.action import ActionClient
from control_msgs.action import FollowJointTrajectory, GripperCommand

class ArmGripperController(Node):
    def __init__(self):
        super().__init__('arm_gripper_controller')
        self.arm_client = ActionClient(
            self, FollowJointTrajectory,
            '/arm_controller/follow_joint_trajectory')
        self.gripper_client = ActionClient(
            self, GripperCommand,
            '/gripper_controller/gripper_cmd')
    
    def move_arm_to(self, joint_positions, duration_sec=2.0):
        # Build JointTrajectoryPoint and send action goal
        ...
    
    def open_gripper(self):
        # Send GripperCommand with position=0.0 (fully open)
        ...
    
    def close_gripper(self):
        # Send GripperCommand with position=0.8 (closed)
        ...
    
    def run_pick_and_place_loop(self):
        while True:
            self.move_arm_to(HOME_POSITIONS)
            self.open_gripper()
            self.move_arm_to(PICK_APPROACH_POSITIONS)
            self.move_arm_to(PICK_POSITIONS)
            self.close_gripper()
            self.move_arm_to(PICK_APPROACH_POSITIONS)  # lift
            self.move_arm_to(PLACE_APPROACH_POSITIONS)
            self.move_arm_to(PLACE_POSITIONS)
            self.open_gripper()
            self.move_arm_to(PLACE_APPROACH_POSITIONS)  # retreat

This script is valuable as a baseline: it proves the controllers work correctly and the hardware interface is functional, even before MoveIt 2 is integrated. It is intentionally simple — no collision avoidance, no workspace monitoring, fixed joint positions — which makes it ideal for initial commissioning and hardware-in-the-loop testing.

Limitation:
Because this script bypasses MoveIt 2 entirely, it has no collision avoidance. If the environment changes (object moved, obstacle added), the robot will attempt to execute the hardcoded trajectory regardless. For production use, all motion must be routed through MoveIt 2 or an equivalent collision-aware planner.

## 6. ROS 2 Communication Architecture

### 6.1 Topics

The following topics are central to system operation:

| Topic | Type | Publisher | Subscriber | Purpose |
| --- | --- | --- | --- | --- |
| /joint_states | sensor_msgs/JointState | joint_state_broadcaster | robot_state_publisher, MoveIt | Real-time joint positions and velocities |
| /robot_description | std_msgs/String | robot_state_publisher | MoveIt, Gazebo | URDF as string |
| /tf | tf2_msgs/TFMessage | robot_state_publisher | MoveIt, RViz | Dynamic transforms |
| /tf_static | tf2_msgs/TFMessage | robot_state_publisher | MoveIt, RViz | Static transforms (world → base_link) |
| /planning_scene | moveit_msgs/PlanningScene | move_group | RViz, MTC | Collision objects, scene state |
| /monitored_planning_scene | moveit_msgs/PlanningScene | move_group | Subscribers | Real-time monitored scene |
| /move_group/status | actionlib_msgs/GoalStatusArray | move_group | Clients | Action server status |
| /arm_controller/joint_trajectory | trajectory_msgs/JointTrajectory | move_group | arm_controller | Planned trajectory to execute |

### 6.2 Services

| Service | Type | Server | Purpose |
| --- | --- | --- | --- |
| /controller_manager/list_controllers | controller_manager_msgs/ListControllers | controller_manager | Query active controllers |
| /controller_manager/switch_controller | controller_manager_msgs/SwitchController | controller_manager | Activate/deactivate controllers |
| /apply_planning_scene | moveit_msgs/ApplyPlanningScene | move_group | Programmatic scene updates |
| /get_planning_scene | moveit_msgs/GetPlanningScene | move_group | Query current scene state |
| /compute_ik | moveit_msgs/GetPositionIK | move_group | Request IK solution |
| /compute_fk | moveit_msgs/GetPositionFK | move_group | Request FK evaluation |
| /check_state_validity | moveit_msgs/GetStateValidity | move_group | Collision check query |

### 6.3 Actions

| Action | Type | Server | Purpose |
| --- | --- | --- | --- |
| /arm_controller/follow_joint_trajectory | control_msgs/FollowJointTrajectory | arm_controller | Execute joint trajectory |
| /gripper_controller/gripper_cmd | control_msgs/GripperCommand | gripper_controller | Command gripper position |
| /move_group | moveit_msgs/MoveGroup | move_group | High-level motion planning + execution |
| /execute_trajectory | moveit_msgs/ExecuteTrajectory | move_group | Execute a pre-planned trajectory |

## 7. ros2_control Integration

ros2_control is the real-time control abstraction layer in ROS 2. Its architecture separates the hardware interface (the plugin that talks to physical hardware or simulation) from the controllers (the algorithms that compute commands).

Component hierarchy:

Controller Manager
├── joint_state_broadcaster (reads state, publishes /joint_states)
├── arm_controller (JointTrajectoryController)
│   ├── Reads: position, velocity state interfaces (6 joints)
│   └── Writes: position command interfaces (6 joints)
└── gripper_controller (GripperActionController)
    ├── Reads: position state interface (finger_joint)
    └── Writes: position command interface (finger_joint)

Hardware Resource Manager:
At the bottom of the stack sits the ResourceManager, which loads the gz_ros2_control/GazeboSimSystem plugin. This plugin bridges the ros2_control command/state interfaces to Gazebo’s joint state publishing and joint command subscription mechanisms. This is the layer where simulation and real hardware are abstracted.

Controller types in detail:

JointTrajectoryController: Accepts FollowJointTrajectory action goals containing multi-point joint trajectories with timing information. Internally it runs a PID loop (or pure position setpoint, depending on configuration) to track the trajectory. It publishes feedback at the controller update rate (typically 500 Hz in simulation).

GripperActionController: A specialized single-DOF controller that accepts GripperCommand action goals specifying a target position and maximum effort. It is simpler than JointTrajectoryController and appropriate for the gripper’s single commanded joint. Its internal control law is a position PID with effort limiting.

JointStateBroadcaster: A passive controller that only reads state interfaces and republishes them on /joint_states. It does not command anything.

## 8. MoveIt 2 Integration

The move_group node is MoveIt 2’s central process. It integrates:

- A planning pipeline (OMPL, PILZ, or other planner plugins)

- A kinematics plugin (KDL or TracIK)

- A collision detection system (FCL — Flexible Collision Library)

- A trajectory execution manager (sends trajectories to ros2_control)

- A planning scene monitor (listens to /joint_states and /tf, maintains world state)

- Capability plugins for IK, FK, state validity checking, and Cartesian planning

Planning pipeline execution flow:

Client Request (C++ / Python / MTC Stage)
    │
    ▼
move_group Action Server (/move_group)
    │
    ├── MotionPlanRequest construction
    │
    ├── Planning Scene Monitor → snapshot of current world state
    │
    ├── Planner Plugin (OMPL / PILZ)
    │   ├── State space definition (joint space for UR3 = R^6)
    │   ├── Constraint checking (joint limits, collision)
    │   ├── IK calls (for Cartesian goals)
    │   └── Returns RobotTrajectory
    │
    ├── Trajectory Processing
    │   ├── Time parametrization (TOTG / Ruckig)
    │   └── Velocity/acceleration scaling
    │
    └── Trajectory Execution Manager
        └── Sends FollowJointTrajectory goal to arm_controller

Time parametrization:
The planned path is initially a sequence of waypoints with no timing information. The TimeOptimalTrajectoryGeneration (TOTG) plugin adds timing by solving a time-optimal trajectory problem subject to per-joint velocity and acceleration bounds (from joint_limits.yaml). The scaling factors set in application code (e.g., setMaxVelocityScalingFactor(0.3)) proportionally reduce these bounds, effectively slowing the motion.

## 9. MoveIt Task Constructor (MTC) Pipeline

MTC is a meta-planner built on top of MoveIt 2. It introduces a compositional task planning paradigm suitable for multi-step manipulation.

Core data structures:

- Task: The root container. Owns a TaskModel (directed acyclic graph of stages).

- Stage: Base class for all planning units. Has input/output interfaces typed as InterfaceState (a robot state + planning scene snapshot).

- InterfaceState: A tuple of (robot_state, planning_scene, properties). Properties allow stages to pass metadata (e.g., grasp frame) downstream.

- SolutionBase: A planned trajectory segment connecting two InterfaceStates. Multiple solutions per stage form an AND/OR search tree.

Stage resolution and the solution tree:

When task.plan() is called, MTC performs a recursive planning traversal:

- Generator stages produce initial InterfaceStates (e.g., CurrentState samples the current robot joint state).

- Forward propagators extend solutions forward in time.

- Backward propagators extend solutions backward (planning from goal to current).

- Connectors bridge forward and backward solutions at their shared boundary.

The result is a solution tree where each leaf represents a complete task execution plan. MTC selects the minimum-cost leaf (cost is typically path length or planning time) and exposes it for execution.

GenerateGraspPose stage:
This generator stage samples candidate grasp poses around the target object. It typically works by:

- Defining a grasping frame (e.g., gripper approach axis = +z)

- Sampling rotations around the object’s vertical axis at a fixed grasp depth

- For each sampled pose, computing IK to check reachability

- Filtering samples that collide with the table or the object itself

- Returning valid grasp InterfaceStates

The number of samples and the angle discretization control the trade-off between planning time and grasp diversity.

Planning scene modification stages:
AttachObject and DetachObject stages call applyPlanningScene to modify the collision environment. Attaching the object to the gripper frame means subsequent collision checking correctly accounts for the grasped object’s geometry as the arm moves to the place location.

## 10. Gazebo Harmonic Integration

Gazebo Harmonic (formerly Ignition Gazebo, now the standalone Gazebo) represents a complete rewrite of Gazebo Classic with a modern, library-based architecture using the gz-sim framework.

Key differences from Gazebo Classic:

| Feature | Gazebo Classic | Gazebo Harmonic |
| --- | --- | --- |
| Architecture | Monolithic process | Multi-process, library-based |
| ROS bridge | gazebo_ros_pkgs | ros_gz bridge |
| ros2_control | gazebo_ros2_control | gz_ros2_control |
| SDF support | SDFormat 1.6 | SDFormat 1.11 |
| Mimic joints | Plugin-based | Physics-level enforcement |
| Rendering | OGRE 1.x | OGRE-Next / Vulkan |

Physics engine:
Gazebo Harmonic uses DART (Dynamic Animation and Robotics Toolkit) as its default physics engine, with optional Bullet and TPE (Trivial Physics Engine) backends. DART provides accurate rigid body dynamics with constraint-based contact resolution, suitable for grasping simulation.

The gz-ros2-control plugin:
The gz_ros2_control/GazeboSimSystem plugin is loaded in the robot’s SDF via the <plugin> tag. At simulation start, it:

- Parses the robot’s URDF for ros2_control <hardware> sections

- Creates StateInterface objects for each declared state interface (joint positions, velocities, efforts)

- Creates CommandInterface objects for each declared command interface

- At each simulation step, copies Gazebo joint states → ROS 2 state interfaces, and copies ROS 2 command interfaces → Gazebo joint commands

This bidirectional copy happens synchronously at the simulation rate, ensuring that ros2_control’s update loop has consistent state information.

## 11. Kinematic Chain and Motion Planning

### 11.1 Forward Kinematics

The forward kinematics (FK) of the UR3 is computed by the robot_state_publisher node, which subscribes to /joint_states and uses the URDF to compute all link transforms, publishing them on /tf. MoveIt 2 maintains its own internal FK computation (via the kinematics plugin) for planning purposes.

The FK computation is a matrix chain multiplication: T_world_EE = T_world_base · T_base_1(θ₁) · T_1_2(θ₂) · T_2_3(θ₃) · T_3_4(θ₄) · T_4_5(θ₅) · T_5_6(θ₆) · T_6_flange · T_flange_gripper_tip

The final two transforms (flange to gripper tip) are fixed and defined by the gripper URDF.

### 11.2 Inverse Kinematics

IK for the UR3 is non-trivial due to the non-spherical wrist. KDL’s iterative solver begins from a seed state (typically the current robot configuration) and iterates:

θ_{k+1} = θ_k + J†(θ_k) · Δx

where J† is the pseudoinverse of the geometric Jacobian and Δx is the Cartesian error between the current and desired end-effector pose. The algorithm terminates when |Δx| < ε or iteration count exceeds a threshold.

Configuration singularities:
The UR3 has singularities in three configurations: 1. Shoulder singularity: Wrist center on the shoulder rotation axis 2. Elbow singularity: Arm fully extended 3. Wrist singularity: Joints 4 and 6 collinear

Near singularities, J becomes rank-deficient and J† becomes ill-conditioned, leading to large joint velocity commands. MoveIt 2 detects proximity to singularities and rejects trajectories that pass too close. The Jacobian condition number threshold is configurable in kinematics.yaml.

### 11.3 Collision Detection

MoveIt 2 uses FCL (Flexible Collision Library) for collision queries. The collision world contains: - Self-collision geometry: Derived from URDF collision meshes - World collision geometry: Objects published to the planning scene - Padding: Each link has a configurable padding value (typically 5mm) to add safety margins

OMPL’s state validity checker calls FCL’s broad-phase and narrow-phase collision detection at each state sample. The broad-phase uses an AABB tree for rapid rejection of non-colliding pairs; the narrow-phase uses GJK+EPA for exact convex collision queries and mesh-mesh checks for non-convex geometry.

## 12. Pick-and-Place Workflow

### 12.1 High-Level Sequence

Phase 1: Initialization
  ├── Gazebo simulation spawned (world + robot)
  ├── ros2_control controllers activated
  ├── MoveIt 2 move_group started
  ├── Planning scene populated (table, pick object)
  └── Robot moved to home configuration

Phase 2: Pre-Grasp
  ├── Gripper commanded to OPEN state
  └── Arm planned to pre-grasp approach pose (above object)

Phase 3: Grasp
  ├── Arm linearly descended to grasp pose (PILZ LIN)
  ├── Gripper commanded to CLOSED state
  └── Object attached to gripper in planning scene

Phase 4: Lift and Transfer
  ├── Arm linearly lifted above table (PILZ LIN)
  ├── Arm planned to pre-place approach pose (above target)
  └── Arm linearly descended to place pose (PILZ LIN)

Phase 5: Place and Retreat
  ├── Gripper commanded to OPEN state
  ├── Object detached from gripper in planning scene
  └── Arm linearly retreated and returned to home

### 12.2 Pose Definitions

All poses in the system are defined in the world frame. The relationship between the robot base and the world is fixed by the world → base_link static transform published at launch.

Pick pose (example values):

position: {x: 0.35, y: 0.0, z: 0.87}  # Above table surface
orientation: {w: 0.0, x: 1.0, y: 0.0, z: 0.0}  # Gripper pointing down

Place pose (example values):

position: {x: 0.35, y: 0.25, z: 0.87}  # Offset in y
orientation: {w: 0.0, x: 1.0, y: 0.0, z: 0.0}  # Same orientation

### 12.3 Cartesian Path Planning Details

The approach and retreat phases use PILZ LIN or computeCartesianPath with a step size of 10mm. At each step, the desired Cartesian pose is computed, IK is solved, and collision is checked. The resulting sequence of valid joint configurations forms the trajectory.

The approach direction is typically along the gripper’s approach axis (−z in the gripper frame, which is the direction from the gripper body toward the finger tips). This is hardcoded in the MTC MoveRelative stage via a DirectionGenerator that specifies the direction vector.

## 13. Gripper Control Architecture

### 13.1 Control Loop

The GripperActionController implements a simple position control loop for finger_joint:

Goal: target_position = 0.8 rad (closed)
         │
         ▼
   PID controller
   error = target_position - current_position
   command = Kp * error + Kd * d(error)/dt
         │
         ▼
   Position command interface → Gazebo joint effort

The position range of finger_joint is 0.0 (fully open, 85mm gap) to 0.8 rad (fully closed, 0mm gap). The GripperActionController reports success when the joint reaches within a position tolerance of the goal, or reports aborted if effort exceeds the maximum effort parameter (simulating a contact event in hardware).

### 13.2 In-Grasp Object Interaction

In simulation, Gazebo Harmonic models grasp contact as a constraint satisfaction problem between the gripper finger collision geometry and the grasped object’s surface. When the gripper closes around the object:

- Contact forces are computed by the DART physics engine

- If contact forces exceed the object’s friction cone, slippage occurs

- The AttachObject call in MTC bypasses this physics interaction by kinematically attaching the object to the gripper frame, effectively simulating a perfect grasp

This is a known limitation of simulated grasping: real-world grasps must account for friction, surface compliance, force sensing, and slip detection. The kinematic attachment approach is appropriate for demonstrating the planning pipeline but does not validate grasp quality.

## 14. Launch Architecture

### 14.1 Launch File Hierarchy

robot.sh (shell script orchestrator)
│
├── ur.gazebo.launch.py (ur_gazebo)
│   ├── gz_sim (Gazebo Harmonic)
│   ├── robot_state_publisher
│   ├── spawn_entity (robot into Gazebo)
│   └── controller_manager + spawners
│
├── move_group.launch.py (moveit_config)
│   ├── move_group node
│   └── (optional) moveit_rviz.launch.py
│
└── ur_mtc_pick_place_demo node (ur_mtc_pick_place_demo)
    └── planning_scene_server node

### 14.2 Parameter Loading Pattern

Each launch file uses the standard ROS 2 Python launch API pattern:

from launch import LaunchDescription
from launch_ros.actions import Node
from launch.actions import DeclareLaunchArgument, ExecuteProcess
from launch.substitutions import LaunchConfiguration, Command

def generate_launch_description():
    robot_description = {"robot_description": 
        Command(["xacro ", urdf_path, " ur_type:=", ur_type_arg])}
    
    return LaunchDescription([
        DeclareLaunchArgument("ur_type", default_value="ur3"),
        Node(
            package="robot_state_publisher",
            executable="robot_state_publisher",
            parameters=[robot_description]
        ),
        # ...
    ])

The xacro processing occurs at launch time (not at build time), enabling runtime parameterization of the robot model (e.g., selecting UR3 vs. UR5 via the ur_type argument).

## 15. Controller Configuration

ur_controllers.yaml (annotated):

controller_manager:
  ros__parameters:
    update_rate: 500  # Hz — control loop frequency

joint_state_broadcaster:
  ros__parameters:
    # No additional params; broadcasts all joints

arm_controller:
  ros__parameters:
    joints:
      - shoulder_pan_joint
      - shoulder_lift_joint
      - elbow_joint
      - wrist_1_joint
      - wrist_2_joint
      - wrist_3_joint
    command_interfaces:
      - position
    state_interfaces:
      - position
      - velocity
    allow_partial_joints_goal: false
    open_loop_control: false  # Use state feedback
    constraints:
      goal_time: 0.5          # Seconds past trajectory end to reach goal
      stopped_velocity_tolerance: 0.01
      shoulder_pan_joint:
        goal: 0.01            # Radians tolerance at goal
        trajectory: 0.1

gripper_controller:
  ros__parameters:
    joint: finger_joint
    allow_stalling: true
    stall_velocity_threshold: 0.001
    stall_timeout: 1.0
    max_effort: 100.0

The open_loop_control: false setting means the arm controller uses closed-loop position feedback. The goal_time tolerance gives the controller a 0.5-second window past the trajectory’s last timestamp to reach the final waypoint — important for simulation where physics lag can cause timing mismatches.

## 16. Data Flow and Runtime Behavior

### 16.1 Steady-State Data Flow at 500 Hz

Gazebo Physics Step (1ms)
    │
    ├── Joint positions → gz_ros2_control state interfaces
    │
    └── gz_ros2_control update()
            │
            ├── controller_manager update()
            │       │
            │       ├── joint_state_broadcaster → /joint_states
            │       ├── arm_controller: read state → compute cmd → write cmd interfaces
            │       └── gripper_controller: read state → compute cmd → write cmd
            │
            └── Command interfaces → Gazebo joint commands

### 16.2 Motion Planning Request Data Flow (one-shot)

Application code calls move_group->plan()
    │
    ├── ROS 2 action goal sent to /move_group action server
    │
    ├── move_group: lock planning scene snapshot
    │
    ├── OMPL planner: sample random configurations, check validity
    │   (FCL collision checks at each sample, ~1000s per planning call)
    │
    ├── OMPL returns path (list of joint configurations)
    │
    ├── TOTG: compute timing for each waypoint
    │
    ├── Action result: RobotTrajectory returned to client
    │
    └── (if execute=true) Trajectory sent to arm_controller via action

### 16.3 MTC Plan-and-Execute Data Flow

task.plan(max_solutions=5)
    │
    ├── Stage 1 (CurrentState): read /joint_states → InterfaceState
    │
    ├── Stage 2 (OpenGripper): IK + OMPL plan → trajectory segment
    │
    ├── ... (intermediate stages) ...
    │
    ├── Cost minimization: select best complete solution
    │
    └── task.execute(solution)
            │
            └── Per stage: call move_group execute → arm_controller / gripper_controller

## 17. Software Engineering Analysis

### 17.1 Strengths

Layered architecture: The system cleanly separates description, simulation, control, planning, and application layers. Each layer can be swapped independently — for example, replacing Gazebo with Isaac Sim, or replacing OMPL with CHOMP, without modifying adjacent layers.

Dual planning paradigm: Supporting both direct MoveGroup API (for simple motions) and MTC (for compositional tasks) gives developers a clear upgrade path. Start simple, add complexity only when needed.

Parametric URDF: Using xacro enables the same description to support multiple UR arm variants and multiple gripper types through launch-time argument substitution. This is significantly better than maintaining parallel URDF files.

Simulation fidelity: Using Gazebo Harmonic (rather than simple joint position publishers) provides realistic physics, contact dynamics, and sensor simulation, enabling genuine sim-to-real transfer research.

Mimic joint implementation: The decision to use Gazebo Harmonic’s physics-level mimic joint enforcement (rather than a controller plugin) is architecturally sound and produces more stable gripper simulation.

### 17.2 Weaknesses and Limitations

Hardcoded pick/place poses: The application-layer scripts use fixed joint configurations or Cartesian poses. Real systems require perception pipelines (RGB-D cameras, point cloud processing, object detection) to compute poses dynamically. The repository acknowledges this and includes in-progress perception work.

Bootstrap delay fragility: The ~40-second fixed sleep in robot.sh before controller spawning is fragile. If the system is slower (resource-constrained machine, cold start), controllers may fail to spawn. A lifecycle-based or watchdog-based approach would be more robust.

No error recovery: The Python loop controller does not implement error recovery logic. If an action goal fails (joint limit hit, collision detected), the script does not gracefully handle the failure and may get stuck. Industrial systems require state machines with recovery behaviors.

MTC planning scene dependency: The MTC pipeline requires the planning scene to contain accurate collision objects (table geometry, object location). If the table geometry in the URDF world file and the planning scene YAML are inconsistent, pick-and-place will fail with false-positive collisions or allow arm-table collisions.

Single-robot, single-object scope: The architecture does not currently support multi-robot coordination or multi-object pick-and-place sequences. Extending to these scenarios would require significant changes to the task planning layer.

### 17.3 Design Decision Analysis

| Decision | Rationale | Trade-off |
| --- | --- | --- |
| Physics-level mimic joints | Gazebo Harmonic compatibility | Less portable to older Gazebo |
| Position command interface (not effort) | Simpler control, sufficient for manipulation | Cannot simulate force control or compliance |
| 500 Hz controller rate | Matches typical UR robot EtherCAT rate | May be too fast for some simulation hosts |
| KDL default IK solver | Zero additional dependencies | Slower than TracIK for the UR3’s non-spherical wrist |
| OMPL RRTConnect default | Empirically fastest for 6-DOF | Non-deterministic planning time |
| Shell script orchestration | Simple to understand and debug | Fragile timing, no fault isolation |
| Python for application layer | Rapid prototyping | Higher latency than C++ |

## 18. Comparison to Industrial Systems (UR5e)

The UR5e is Universal Robots’ collaborative industrial variant with an integrated force/torque sensor, improved I/O, and polyscope-based programming. Comparing this repository’s architecture to real UR deployment reveals instructive gaps:

| Aspect | This Repository | Real UR5e Deployment |
| --- | --- | --- |
| Robot Description | Custom URDF/xacro | Official ur_robot_description pkg |
| Hardware Interface | gz_ros2_control (simulation) | ur_robot_driver (EtherCAT) |
| Trajectory Execution | ROS 2 action (sim) | RTDE + FollowJointTrajectory |
| Force Control | Not implemented | Force mode, compliance control |
| Safety Monitoring | None (simulation) | Safety planes, joint torque limits |
| E-stop | None | Hardware E-stop, safety PLC |
| Calibration | Default URDF | Kinematic calibration file |
| Gripper Protocol | ros2_control position | Modbus RTU or URCap |
| Production Speed | 20-30% (demo) | 80-100% with safety assessment |

The official ur_robot_driver package (maintained by Universal Robots) provides a complete production-grade interface to real UR hardware. This repository’s architecture is compatible with that driver’s interface (using the same controller types and action servers) and would require primarily the hardware interface swap to run on real hardware.

Key real-world considerations not addressed:

- Kinematic calibration: Real UR robots have per-robot calibration offsets (stored on the controller) that must be loaded into the URDF for accurate positioning. Without calibration, TCP position error can be 1-2mm, which is significant for precision assembly.

- Force/torque sensing: The UR5e’s built-in F/T sensor enables compliant grasping, contact detection, and force-limited insertion. This repository’s simulation does not model force feedback at the gripper.

- Speed and safety: Industrial deployment requires a formal risk assessment. The robot’s speed is limited not arbitrarily but based on separation distance, payload, and stopping time calculations.

- Network reliability: Real UR communication uses a 1kHz EtherCAT bus. Network latency affects trajectory tracking accuracy and safe stopping capability.

## 19. Alternative Implementation Approaches

### 19.1 Alternative Motion Planning Frameworks

| Framework | Characteristics | When to Choose |
| --- | --- | --- |
| MoveIt 2 + OMPL (current) | Sampling-based, probabilistically complete | General 6-DOF manipulation |
| MoveIt 2 + PILZ | Deterministic, Cartesian | Industrial straight-line motions |
| Drake | Trajectory optimization, contact-aware | Research, contact-rich manipulation |
| LOCO | Learning-based motion generation | Sim-to-real with learned priors |
| cuRobo | GPU-accelerated motion planning | High-frequency replanning, grasping |

### 19.2 Alternative Task Planning Frameworks

| Framework | Characteristics | When to Choose |
| --- | --- | --- |
| MTC (current) | Compositional, MoveIt-native | Multi-stage manipulation |
| BehaviorTree.CPP | Behavior tree reactive planning | Sensor-reactive behaviors |
| SMACH (ROS) | State machine hierarchies | Well-defined task sequences |
| PDDLStream | Logic-based task + motion planning | Complex combinatorial tasks |
| LangChain + LLM | Natural language task specification | Human-robot interaction |

### 19.3 Alternative Simulation Environments

| Environment | Characteristics | When to Choose |
| --- | --- | --- |
| Gazebo Harmonic (current) | Open-source, ROS-native | Research, education |
| Isaac Sim (NVIDIA) | GPU-accelerated, photorealistic | Sim-to-real with synthetic data |
| MuJoCo | Fast physics, RL-friendly | Reinforcement learning |
| PyBullet | Lightweight, Python-native | Rapid prototyping |
| Webots | Educational, multi-robot | Swarm robotics |

### 19.4 Alternative Gripper Interfaces

The current position-based GripperActionController approach can be augmented with:

- Force-based grasping: Using JointEffortController and force sensors to detect contact and apply stable grasp forces

- Adaptive grasping: Using the Robotiq driver’s full API (via Modbus) for finger force, speed, and position control independently

- Suction gripper: Replacing the parallel gripper with a vacuum-based end effector, requiring different contact modeling and grasp planning

## 20. Emerging Capabilities and Future Directions

The repository documents several in-progress extensions that represent the intended evolution of the system:

### 20.1 Reinforcement Learning Integration

A Soft Actor-Critic (SAC) policy trained in MuJoCo is being transferred to Gazebo for deployed inference. The RL policy observes joint positions, end-effector pose, and object pose (from ground truth or perception), and outputs joint velocity commands. The training pipeline uses Stable-Baselines3 with a 5-phase curriculum:

- Reach (arm approaches object)

- Pre-grasp (arm positions above object)

- Grasp (close gripper around object)

- Lift (raise object above table)

- Place (move object to target and release)

Key challenge: MuJoCo-to-Gazebo transfer. Physics differences (contact models, inertia estimation accuracy, friction coefficients) cause policy performance degradation. Domain randomization during training is the standard mitigation.

### 20.2 Perception Pipeline

An Intel D435 depth camera provides RGB-D point cloud data for object detection and grasp pose estimation:

- Color-based detection: HSV color thresholding to isolate objects by color

- YOLO object detection: ultralytics YOLOv8 for bounding box detection in the RGB image

- PCL clustering: Euclidean cluster extraction from the depth point cloud to obtain object 3D position

- Grasp pose estimation: From the clustered point cloud, compute grasp approach axis and position

This perception pipeline would replace the hardcoded pick poses in the current scripts, enabling truly autonomous manipulation of previously unseen object positions.

### 20.3 LLM-Based Task Planning

An Ollama-hosted local LLM (e.g., LLaMA3 or Mistral) receives natural language task descriptions and generates structured task specifications that are then executed by the MTC pipeline or direct controller commands. This is an early-stage integration that demonstrates the architectural extensibility of the system.

## 21. Conclusions

The darshmenon/UR3_ROS2_PICK_AND_PLACE repository represents a well-structured, educationally valuable implementation of a complete robotic manipulation stack using modern ROS 2 tooling. Its greatest strengths are its clear layered architecture, its dual approach to motion planning (direct MoveGroup and MTC), and its honest documentation of known limitations and future directions.

For a learner studying this system, the recommended path is:

- Start with ur_description — understand the URDF model and xacro parameterization

- Study ur_gazebo — understand how simulation, ros2_control, and the controller manager are launched and wired together

- Examine moveit_config — understand SRDF, kinematics configuration, and planner selection

- Run and study ur_system_tests/arm_gripper_loop_controller.py — understand direct controller communication

- Study ur_moveit_demos/custom_zigzag_motion — understand the MoveGroup C++ API

- Study ur_mtc_pick_place_demo — understand compositional task planning with MTC

For someone designing a similar system, the key architectural lessons are:

- Separate description, simulation, control, planning, and application into distinct packages

- Use parametric xacro models rather than multiple hardcoded URDFs

- Implement mimic joints at the physics level in Gazebo Harmonic rather than the controller level

- Support multiple planners (OMPL for free-space, PILZ for Cartesian)

- Build toward MTC from the beginning; retrofitting MTC onto a MoveGroup-only architecture is non-trivial

- Always provide a direct controller bypass path for debugging and commissioning

The repository’s in-progress extensions — RL integration, depth camera perception, and LLM task planning — represent a thoughtful roadmap toward a fully autonomous manipulation system. Completing the perception-to-grasp pipeline would elevate this from a compelling demonstration to a genuine research platform for sim-to-real transfer in robotic manipulation.

This technical analysis was prepared as a comprehensive case study based on examination of the public repository structure, documentation, and companion materials. All architectural claims are grounded in the observable code structure, ROS 2 and MoveIt 2 documentation, and established robotics engineering principles.